ILO-1. Immersive Modeling for Lake Mead based on the Principle of Division of Reservoir Inflow

Learning Objectives

- Describe the core components of systems optimization problems: the dimensions, decisions, objectives, and constraints.

- Formulate, solve, and visualize results for problems with multiple objectives including economic, environmental, social, aesthetic, etc.

- Interpret solutions and describe new insight(s) to improve system management, design, and planning that derive from the results.

- Develop solutions that adapt to deep uncertainties about future conditions.

- Present your work to technical and non-technical audiences in common formats required of practicing engineers, including written reports, oral presentations, and online data/model repositories.

- Immerse and have fun with this activity.

# Introduction

The Governors of California, Nevada, Arizona, President of Mexico, U.S. Bureau of Reclamation, and Council Leaders of our First Nations have appointed your group members to negotiate new reservoir operations for Lake Mead that are more responsive to declining water availability. Experts tell us that water availability in the Colorado River will decline and become more volatile due to drought, aridification, and declining soil moisture. Your instructions are to increase managers flexibility to conserve and consume water more autonomously of other parties. Additionally, your supervisors want to reduce water scarcity, protect Lake Mead from falling below critical levels, and preserve the status quo for the native, endangered fish of the Grand Canyon compared to the existing operations. You are to focus your work for scenarios of extreme low reservoir inflow and low storage.

I have provided an immersive online collaborative model for a Lake Mead that is based on the principles of divide reservoir inflow, subtract evaporation, and users consume and conserve within their available water and in response to their available water other’s decisions, and the real-time discussion of decisions (Rosenberg, 2025). Each student will immerse in and personify one (or more) water users who represent:

- California – Imperial Irrigation District and Metropolitan Water District of Southern California.

- Arizona – Primarily the Central Arizona Project (CAP).

- Nevada – Southern Nevada Water Authority (i.e., Las Vegas).

- Mexico.

- Tribal Nations of the Lower Basin (principally in Arizona).

- U.S. Bureau of Reclamation – Protect Lake Mead from drawdown below critical levels.

Find more information and background about existing Colorado River management in Section 4 below.

# Directions

- Navigate to the model repository on Github - https://github.com/ImmersiveModelsColoradoRiver/LakeMeadDivideInflow.

- Scroll down to read more about the Purpose, Key Ideas, and Directions.

- Click the link to the Model File in the Course Schedule (Google Sheets file).

- Follow the remaining directions in the ReadMe-Directions worksheet.

- Select one person to additionally take notes of the discussion during the session. Try as best as possible to record the conversation - write down what people actually say as they go through the session (akin to a transcript). Ask people to clarify why they made the choices they did. If you need more time, to keep up with the conversation, ask people to pause or repeat what they said. Find a blank template version of the notes here:

  - ImmersiveModel-RunningNotes-LakeMead-Template.docx

- Set the starting Lake Mead elevation 1,025 feet (on the Master Worksheet in Cell B19). This low storage has yet to be seen in 100+ years.

- Complete 5 years (Master worksheet - Columns C to G). Select scenarios of extreme low inflow. Set the formula on the Master worksheet Cell C28 to point to Cell F8 on the last worksheet LakeMeadInflowScenarios.

  - For each next year, copy the formula on the Master worksheet in Cell C28 into the next column. Reveal each new inflow, year by year.

- At end of 5 years, go around, each person answers the questions at the end --

- Which new insights did you take away from the model session?

- Which things did you like, which things to improve?

# Deliverables.

Each student in their own 1-page report:

- Introduce the situation - describe the problem the model seeks to provide insight on (Grading Rubric – Technical Correctness 10 pts).

- Describe your management strategy for your party, how the strategy differed from current operations, and how your strategy evolved as model years progressed (Grading Rubric – Technical Correctness 15 pts).

- Describe how the modeled Lake Mead inflow differed from historical Lake Mead inflow (Grading Rubric – Technical Correctness 5 pts).

- List 2 or more insights you gained from the immersive modeling exercise (20 pts).

- Recommend what action your superior should take for new operations post-2026 (15 pts).

- Provide an appendix that shows key model results (Table 1) (5 pts).

Additional Notes

- 1-page reports may include a few extra pages for appendices, e.g., table(s) and/or figure(s) to summarize findings.

- For example, include a table for each year that shows Lake Mead Inflows, Available Water, User’s Purchases/Sales – both volume and prices, consumptive use, and end-of-year Lake Mead storage.

- See Help for writing a 1-page report on the Course Front Page under Materials and Grading Rubrics for Course Work.

- Fill out and attach the Grading Rubric for HW-1 to the end of your report (see final 2 pages of this prompt)

- Also submit the completed spreadsheet model file on Canvas – each student submits a copy of the model file for the session they participated in.

Box 1. Synopsis of Current Colorado River Operations

- 1922, 1928, 1944, 1956, 1964, and 1968 --  https://www.usbr.gov/lc/region/g1000/lawofrvr.html

- 2007 Interim Guidelines. Lower Basin states increase mandatory conservation as Lake Mead level falls from 1,075 to 1,025 feet; Intentionally created surplus (aka conservation) accounts in Lake Mead for Lower Basin states (Section 3); Equalize storage in Lake Powell and Lake Mead (Section 6). https://www.usbr.gov/lc/region/programs/strategies/RecordofDecision.pdf.

- 2012 and 2017. Minutes 319 and 323 to the 1944 US-Mexico Treaty. Mexico increases mandatory conservation as Lake Mead’s level falls from 1,090 to 1,025 feet. https://www.ibwc.gov/Treaties_Minutes/Minutes.html.

- 2018 Ten Tribes Partnership Water Study. Quantified 2.0 million acre-feet (maf) rights in Upper and Lower Basins and 0.8 maf claims. https://www.usbr.gov/lc/region/programs/crbstudy/tws/finalreport.html.

- 2019 Upper Basin Drought Contingency Plan. Protect Lake Powell elevation of 3,525 feet (5.9 maf). Prevent Lake Powell to fall to minimum power pool elevation of 3,490 feet (4.0 maf). https://www.usbr.gov/dcp/finaldocs.html.

- 2019 Lower Basin Drought Contingency Plan. Increase mandatory conservation targets as Lake Mead’s level falls from 1,090 feet to 1,025 feet. See current mandatory conservation schedule in (Castle and Fleck, 2019). Protect Lake Mead from falling below elevation 1,020 feet. https://www.usbr.gov/dcp/finaldocs.html.

- 2026. Interim Guidelines and Drought Contingency Plans expire.

Current Colorado River operations

The Colorado River basin has a long history. The parties do not get along. There is much written material. Here I provide key pieces:

- Map showing Upper Basin, Lower Basin, Glen Canyon Dam/Lake Powell, Hoover Dam/Lake Mead, and diversions inside and outside the hydrologic basin (USBR, 2012).

- Synopsis of Current Colorado River operations with links to the legal documents (Box 1).

- Castle and Fleck (2019):

  - Summarize current Colorado River operations in slightly more detail than Box 1.

  - Describe what happens when the Upper Basin is unable to deliver 8.23 million acre-feet (maf) of water per year to Lower Basin as required in the 1922 Compact and 1944 US-Mexico Treaty.

- USBR (2021) show unregulated inflows to Lake Powell. The interactive model uses “natural” flow. To convert “unregulated” to “natural” flow, add ~ 4.0 maf representing Upper Basin consumptive use without reservoirs.

- Salehabadi et al. (2020) provide more depth on hydrologic scenarios with natural flow.

- Rosenberg (2023) provides:

  - Excel version of the interactive model.

  - Slides of key concepts.

- Kuhn and Fleck (2019) give a well written history of Colorado River management. Read this piece for fun or to go in depth on a particular piece of management.

Box 2. Start-Up Directions

| - Download Excel file from repository. |  |
| --- | --- |
| - Move file into your Google Drive. |  |
| - Open as a Google Sheet. |  |
| - Invite participants. |  |
| - Select ReadMe-Directions sheet. Follow directions. |  |

Download and Use the Interactive Model

Box 2 provides the directions to download and use the interactive model for a combined Lake Powell-Lake Mead system (Rosenberg, 2023). Rosenberg (2023) also provides the download link to the Excel file and repository, slides of key features, and directions.

# References

Castle, A., and Fleck, J. (2019). "The Risk of Curtailment under the Colorado River Compact." SSRN eLibrary. http://dx.doi.org/10.2139/ssrn.3483654.

Kuhn, E., and Fleck, J. (2019). Science Be Dammed: How Ignoring Inconvenient Science Drained the Colorado River, University of Arizona Press.

Rosenberg, D. E. (2023). "Colorado River Basin Water Accounts: Provoke discussion about more adaptive reservoir operations." Hydroshare.org. https://doi.org/10.4211/hs.090be1e69e734c8991a1c4d8f9b9e21c.

Rosenberg, D. E. (2025). "Immersive Model: Lake Mead based on the Principle of Divide Reservoir Inflow." Github, https://github.com/dzeke/ColoradoRiverCollaborate/tree/main/LakeMeadWaterBankDivideInflow.

Salehabadi, H., Tarboton, D., Kuhn, E., Udall, B., Wheeler, K., E.Rosenberg, D., Goeking, S., and Schmidt, J. C. (2020). "Stream flow and Losses of the Colorado River in the Southern Colorado Plateau." Center for Colorado River Studies, Utah State University, Logan, Utah. https://qcnr.usu.edu/coloradoriver/files/WhitePaper4.pdf.

USBR. (2012). "Colorado River Basin Water Supply and Demand Study." U.S. Department of Interior, Bureau of Reclamation, Washington, D.C., https://www.usbr.gov/lc/region/programs/crbstudy.html.

USBR. (2021). "Lake Powell Unregulated Inflow." https://www.usbr.gov/uc/water/crsp/studies/images/PowellForecast.png [Accessed on: September 28, 2021].

| Category (Max. Score) | No Evidence | Doesn’t Meet Standard | Nearly Meets Standard | Meets Standard | Exceeds Standard | Self- Score | Instructor Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Title (1) | Absent 0 | Evidence of two or less 0 | Evidence of three 0 | Evidence of four 1 | Title – can assess main point from title alone; Name, Instructors’ Names, Course, Date, Neatly finished 1 |  |  |
| Introduction (3) | Absent, no evidence 0 | There is no clear introduction or main topic. 1 | Introduction states the main topic but either: - Does not give a full overview, Or: - Too detailed, leading to annoying repetition later. 2 | The introduction states the main topic and previews the structure of the report. 2 | The introduction states the main topic and previews the structure of the report. Good overview of the problem and solution approach. Gives enough detail to motivate the reader to continue reading. 3 |  |  |
| Organization and structural development of the idea: procedure, results, conclusions (10) | No content provided. 0 | Paragraphs fail to develop the main idea. No section headers or guide to help the reader understand how material is organized. 1 – 5 | Organization of ideas not fully developed. Paragraphs lack supporting detail sentences. No transitions and/or ineffective section headers. 6 - 7 | Paragraph development present but not perfected. Each paragraph has sufficient supporting detail sentences. Few transitions. 8 | Writer demonstrates logic and sequencing of ideas through well-developed section headers, paragraphs, and transitions. The first sentence of each paragraph is the summary sentence. 9 - 10 |  |  |
| Technical Correctness (70) | Questions not addressed. 3 – 42% | The writer has no clue what they are talking about. 45 – 58% | Sketchy: left out required design points. Did not work on this as much as you should have, and it shows. Many important answers are incorrect. 61 – 79% | Discussion lacks adequate detail, but all the necessary points are covered and nearly all answers are correct. 82 – 88% | Provides what was explicitly asked for. The function of each piece is demonstrated to the reader in adequate, but not overwhelming, detail. Answers are correct and reasonable. 91 – 100% |  |  |
|  | - Introduce the problem (10 pts) |  |  |  |  |  |  |
|  | - Describe management strategy, how differed from current operations, and how strategy evolved (15 pts) |  |  |  |  |  |  |
|  | - Describe how the modeled Lake Mead inflow differed from historical Lake Mead inflow (5 pts) |  |  |  |  |  |  |
|  | - List 2 or more insights from the immersive model session (20 pts) |  |  |  |  |  |  |
|  | - Recommend what action your superior should take for new operations post-2026 (15 pts) |  |  |  |  |  |  |
|  | - Table of key model results for each year (5 pts) |  |  |  |  |  |  |

| Category (Max. Score) | No Evidence | Doesn’t Meet Standard | Nearly Meets Standard | Meets Standard | Exceeds Standard | Self- Score | Instructor Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Word Usage and Format (7) | Not applicable | Numerous and distracting errors in punctuation, capitalization, spelling, sentence structure, word usage, significant figures, tables, and figures. Data vomited onto page(s). Unacceptable / unprofessional at the graduate level. 1 – 5 | Misspelled words, poor English grammar and word choice. Main body of report is either longer or significantly less than one page. Figures are too small and/or under-labeled, although they are usually of acceptable quality and focus. Tables incoherent or not cohesive. Bad font sizes. Too much or too little data in appendices. Could be improved by being more meticulous. 5 | Almost no errors in punctuation, capitalization, spelling, sentence structure, word usage, significant figures, and presentation of figures, tables, and appendices. Main body of report is one page or less 6 | Punctuation, capitalization, spelling, sentence structure, word usage, and significant figures all correct. Main body of report is one page or less. Clear, consistent fonts. Good word processing skills. Figures have adequate contrast. Informative figure and table titles and legends. Figures have appropriate axis tick spacing, labels, units, and legends. Table columns cohesive, labeled, and specify units. Document is stapled. Appendices, if provided, are separated by topic, and each have a title, discussion, and proper formatting and display of information 7 |  |  |
| Conclusion (4) | Absent 0 | Incomplete and/or not focused. 1 | The conclusion does not adequately restate the main results. 2 | The conclusion restates the main results. 3 | The conclusion restates the main results, and is an effective summary. 4 |  |  |
| Use of AI (3) | Absent (Return without grade) |  |  | See Policy on Use in Syllabus If used, list the AI tool], provide a short description of use, how use is within the course policy, and link to the transcript of student-AI interactions. If not used, write “No AI was used in the analysis or write-up of this report.” 3 |  |  |  |
| References (2) | Absent 0 | Numerous errors, off-the-wall sources used. 0 | Some errors in citing format; more sources should be cited. 1 | Prior work cited with few errors. 2 | All prior work and data sources are cited in the correct format with no errors. 2 |  |  |
| TOTAL (100) |  |  |  |  |  |  |  |