CEE 6410

Response J – Watershed Area of Suitable Habitat (WASH) (Nonlinear Optimization)

How does the area of suitable aquatic, floodplain, and wetland habitat in the Lower Bear River change if the Hyrum and Porcupine reservoirs fill in 15% with sediment? How do reservoir storage and release and revegetation operations change when reservoirs fill 15% with sediment?

There is no need to submit a full report. Instead briefly describe the problem, how you changed the model input data, and compare results for the base case and new scenario. Provide a link on your Github account where the new input file, .gms file, .lst file, and other output files are located.

Bonus: describe how the solution found using a local solver (Conopt) differs from a solution found using a global solver (BARON). Baron can take hours to solve (run overnight).

Resources:

- This is a big model with lots of companion input data, results, and visualization tools! It will take some time to get into. We have that time in class.

- A paper describing the model and its use is available at https://doi.org/10.1016/j.envsoft.2020.104643.

- A map shows a model schematic (nodes and links) as Appendix A at https://doi.org/10.1016/j.envsoft.2020.104643.

Instructions

- Download all files from the Github Model Repository at https://github.com/ayman510/WASH.

- Follow the Github installation and run instructions (at the bottom of the main page) except, instead:

Step 1: Ignore, you already have GAMS and a license installed!

Step 2: Download the entire repository.

Step 3: Unzip and place it in a folder on your local machine.

Step 4: Download the WASH-CEE6410.gms file available on the CEE 6410 Github (fixes some errors in reading input data and exporting results). Place this file in the subdirectory /InputDataModel.

Step 5: Skip. Note the non-linear program (NLP) solver is defined on line 541 in the file WASH-CEE6410.gms.

Step 6. Open WASH-CEE6410.gms. Read through the introduction and instructions to learn how to read and edit the code, if necessary. Key new concepts:

- Code on lines 93 to 192 defines the sets, parameters, and scalers but does not specify their values.

- Code on lines 216 and 238 to 301 read in data from the Excel file WASH_1yr_InputData.xlsx in the same /InputDataModel subfolder.

- Go ahead and open the file WASH_1yr_InputData.xlsx and see how it is organized. The first Table of Contents worksheet summarizes all the other worksheets and provides a hot link to move directly to that worksheet.

- Back in the file WASH-CEE6410.gms, Code on lines 573 to 579 export all input data and results to the Excel File WASH-CEE6410-Solution.xlsx.

- Code after line 581 is for sensitivity and multi-objective analysis and is commented out.

Steps 7-8. Skip for now.

- RUN THE MODEL to confirm everything works before changing input data in the next step!!

- Enter and change model input data for a new scenario with reservoirs filling in with sediment. To modify the input data and output results it is suggested to do the following:

- Copy and paste the input data file WASH_1yr_InputData.xlsx.

- Give the new input data file a new name such as WASH_1yr-InputData-Sed.xlsx.

- Copy line 216 in WASH-CEE6410.gms. Paste as a new line and change the first input=_____ to be the new file name you created in step b.

- On lines 574 to 579, similarly change the output file names for both the .gdx files

- In your new Excel WASH input data file you created in Step 6b, change reservoir capacity data on the capacity worksheet.

- Run the model again.

- If you run into peculiarities, odd behavior, or get lost in the input or output data files, or code, speak up!

Breakdown of Points

- Overview the problem (10 pts).

- Describe changes to model inputs to represent reservoirs filling with sediment (10 pts).

- Results - How does the area of suitable aquatic, floodplain, and wetland habitat in the Lower Bear River change with reservoirs filling with sediment (10 pts).

- Link to Github where to find new input file, .gms file, .lst file, and other output files are located (10 pts)

- Bonus (20 pts): Describe how the solution found using a local solver (Conopt) differs from a solution found using a global solver (BARON). Baron can take hours to solve (run overnight).